# Technical Documentation: IoT Soil Monitoring System

1. Introduction

The IoT Soil Monitoring System has been developed as part of the ARC Smart Agriculture initiative, with the objective of measuring and transmitting essential soil health parameters in real time. The system combines hardware and software components to facilitate the collection, processing, and visualisation of soil data, thereby supporting more informed agricultural decision-making.

This document offers a comprehensive technical overview of the system architecture, hardware design, software implementation, communication protocols, calibration procedures, and the collaborative efforts undertaken with the Data Management team during development and integration.

2. System Overview

The system architecture comprises an ESP32 DevKitC microcontroller, a 7-in-1 RS485 Modbus soil sensor, and a 16x2 I2C LCD display. Soil sensor data is collected, processed, and displayed locally. Subsequently, the information is transmitted via Wi-Fi to a Raspberry Pi, which serves as the central MQTT broker for data storage and dissemination to the frontend dashboard.

3. Hardware Architecture

3.1 Components

| Component | Function |
| --- | --- |
| ESP32 DevKitC | Main microcontroller for data acquisition, processing, display control, and MQTT transmission. |
| RS485 Modbus Soil Sensor | Measures soil moisture, temperature, pH, electrical conductivity (EC), and nutrient levels (N, P, K).. |
| RS485 Module | Enables serial communication between the sensor and ESP32 over long distances using the Modbus RTU protocol. |
| 16x2 LCD Display (I2C) | Displays soil data in real time. |
| 12V DC Power Supply | Powers the soil sensor.. |
| USB-C Data Cable | Powers the ESP32 and allows firmware upload via Arduino IDE. |
| 3D-Printed Enclosure | Protects the hardware from environmental damage while maintaining airflow and accessibility.. |

3.2 Circuit Wiring

The wiring follows the configuration shown in the system schematic:

- ESP32 → RS485 Module

  - TX → DI

  - RX → RO

  - DE → GPIO 5

  - RE → GPIO 4

  - GND shared between ESP32, RS485, and 12V adapter

- RS485 Module → Soil Sensor

  - A+ → Yellow wire

  - B– → Blue wire

  - VCC → 12V DC adapter

  - GND → Common ground

- ESP32 → LCD Display (I2C)

  - SDA → GPIO 21

  - SCL → GPIO 22

  - VCC → 5V

  - GND → Common ground

All grounds are connected to ensure stable communication between devices. The 12V power supply feeds the sensor, while the ESP32 and LCD are powered via 5V USB input.

4. Software Architecture

4.1 Development Environment

The firmware was developed and uploaded using the Arduino IDE.

The following libraries were required:

- WiFi.h — enables ESP32 network connectivity.

- PubSubClient.h — handles MQTT publishing.

- ModbusMaster.h — communicates with the RS485 Modbus sensor.

- Wire.h — controls I2C communication.

- LiquidCrystal_I2C.h — interfaces with the 16x2 LCD display.

4.2 Firmware Functionality

The ESP32 firmware performs several core functions:

- Establishes Wi-Fi connection using predefined SSID and password.

- Reads soil parameters from the Modbus registers (0x0000 to 0x0007).

- Converts raw sensor data into readable values (°C, %, mg/kg).

- Displays soil parameters on the LCD screen in two cycles, each lasting 5 seconds.

- Publishes all readings in JSON format to the MQTT topic:
arc/soil-monitoring/group7/soil-sensor

- Turns off the LCD backlight for 10 seconds between readings to conserve power.

- Maintains MQTT connection and retries upon disconnection.

Example MQTT payload:

4.3 Communication Protocols

RS485 Modbus RTU

- Baud Rate: 4800

- Slave ID: 1

- Registers: 0x0000 – 0x0007 (seven parameters)

- Provides reliable long-distance serial communication.

MQTT (Message Queuing Telemetry Transport)

- Broker: Raspberry Pi running Mosquitto

- Port: 1883

- Topic: arc/soil-monitoring/group7/soil-sensor

- Chosen for lightweight, low-latency IoT data transfer.

5. Calibration and Accuracy

Sensor calibration was critical to ensure data reliability.
Calibration constants were defined in the firmware:

Each calibration factor was verified using controlled soil samples.

For example, if a moisture reading was 10% below the verified laboratory value, the MOISTURE_CAL constant was adjusted to 1.1.

After adjustments, multiple trials were conducted to confirm stable readings across soil types.

Accuracy verification was performed by comparing live data from the sensor with reference measurements taken using standard agricultural testing kits.

6. Raspberry Pi Integration

The Raspberry Pi 4 served as the data aggregation hub.
It hosted a Mosquitto MQTT broker responsible for receiving and forwarding sensor data to Group 5 (Data Management team).

Process:

- The ESP32 published readings to the MQTT broker.

- The Raspberry Pi stored data locally and exposed it via an Oracle APEX REST API endpoint:

https://oracleapex.com/ords/g3_data/groups/data/7

- This endpoint provided real-time JSON-formatted soil readings to the frontend dashboard.

7. Collaboration with the Data Management Team

Close coordination with Group 5 (Data Management) was essential to ensure reliable system integration.

- Initial integration meeting took place on Wednesday, where data exchange protocols and endpoint structures were agreed upon.

- The Data Management team confirmed API delivery by the end of the following Monday.

- Our team tested API endpoints for stable connectivity and verified correct JSON data formatting from the Raspberry Pi broker.

- Once integration was established, the frontend team (Group 3) could successfully fetch live readings via the APEX API.

- Collaboration included:

  - Aligning MQTT topic structure and data payload format.

  - Testing data flow from ESP32 → MQTT → Raspberry Pi → Oracle APEX API → AgriSense dashboard.

  - Debugging missing or malformed values through shared testing sessions.

This teamwork ensured smooth communication between hardware, backend, and frontend systems.

8. Frontend Integration (AgriSense V2)

The soil monitoring system feeds into the AgriSense V2 dashboard, developed using:

- React + TypeScript

- Tailwind CSS for responsive UI

- Recharts for data visualization

- Google Gemini API integration for AI-driven soil insights

The dashboard retrieves data through the Oracle APEX API and provides:

- Real-time soil data visualization

- Historical charts

- AI-generated crop recommendations

- Interactive “Ask AI” chatbot for context-aware soil and weather queries

This integration converts sensor data into actionable information for farmers and researchers.

9. 3D Printed Enclosure

A custom 3D-printed enclosure was designed to protect the circuit.
The enclosure shields the ESP32 and RS485 connections from moisture and dust while maintaining proper ventilation. It also allows easy maintenance and cable management.

10. Testing and Validation

The team conducted both unit tests and integration tests:

| Test type | Description | Outcome |
| --- | --- | --- |
| Sensor Accuracy Test | Compared sensor readings with lab-tested soil samples. | Within ±5% accuracy. |
| Communication Test | Verified Modbus and MQTT stability under continuous operation. | Stable for 8-hour test cycle. |
| API Test | Tested data flow from ESP32 to Oracle APEX endpoint. | Successful transmission every 15 seconds. |
| UI Integration Test | Ensured AgriSense V2 dashboard displayed correct data. | All readings aligned with database entries. |

11. Conclusion

The IoT Soil Monitoring System demonstrates a robust end-to-end IoT architecture that combines embedded hardware, wireless networking, and intelligent data visualization.

Through collaborative integration with the Data Management and Frontend teams, the system successfully streams accurate soil health data to a web-based dashboard for real-time monitoring and AI-assisted recommendations.

The calibration process verified sensor reliability, and the modular design ensures scalability for future agricultural applications.